AUTONOMOUS MULTI-SENSOR COMMAND SYSTEM

For Unmanned Battle Zones

Predmet: Projektovanje softvera

Studijski program: Master studije — Softversko inzenjerstvo

Akademska godina: 2025/2026

Datum: Mart 2026

# I. UVOD

## 1.1 Opis problema

Savremeno borbeno okruženje karakteriše izuzetna dinamičnost, heterogenost pretnji i neophodnost donošenja odluka u realnom vremenu. Uvođenje autonomnih sistema — UAV (bespilotne letjelice) i UGV (bespilotna kopnena vozila) — omogućava operativan pristup opasnim terenima bez izlaganja ljudskog osoblja, ali ovi sistemi generisu ogromne kolicine heterogenih senzorskih podataka koje je teško integrisati u realnom vremenu i na osnovu kojih treba donositi pouzdane odluke u vremenski kritičnim situacijama.

Ključni problemi koje sistem mora rešiti:

- Fragmentiranost podataka: svaki senzor (akustika, sezmika, radar, LiDAR, kamera...) daje parcijalni pogled na situaciju na terenu

- Latencija: obrada podataka na centralnom serveru uvodi kašnjenja koja mogu biti kritična u borbenim uslovima

- Identifikacija meta: potreba za AI/ML modelima koji pouzdano identifikuju pretnje uz minimalan broj laznih uzbuna

- Procena trajektorije: praćenje mete u realnom vremenu i predvidjanje pozicije za presretanje

- Odluka operatera: zaštita principa covjek-odlučuje (Human-in-the-Loop) — operator mora odobriti svaku odbrambenu akciju

## 1.2 Cilj sistema — 3 kontrolera

Autonomous Multi-Sensor Command System (AMCS) projektovan je kao troslojna arhitektura temeljena na tri međusobno povezana kontrolera, od kojih svaki ima jasno definisanu odgovornost:

Controller 1 — Sensor Detection & Pre-Processing

Senzori (akustika, seizmika, radar, LiDAR, kamera) detektuju aktivnost u monitoriranim zonama. Edge procesor vrši preliminarnu obradu i filtriranje šuma direktno na UAV/UGV platformama, čime se smanjuje propusni opseg prema centralnom serveru za oko 80%. Sistem prati kretanje UGV/UAV jedinica i šalje unapred procesirane podatke na Controller 2.

Controller 2 — Sensor Fusion & Alarm System

SensorFusionEngine prima pre-procesirane podatke i primjenjuje Kalman filter za fuziju heterogenih detekcija. AI/ML model (ThreatDetectionModule) identifikuje metu i procenjuje nivo prijetnje. Target-Tracker inicijalizuje i odrzava pracenje mete. TrajectoryEstimator procjenjuje trajektoriju i tačku presretanja. AlarmSystem generise alarm za operatera kada je pouzdan identifikovana meta (confidence > 0.85).

Controller 3 — Operator Decision (Approve/Deny)

Operater pregleda alarm i podatke o meti, te donosi odluku APPROVE ili DENY. Odobrenje aktivira DefenceController (wakeup, get-ready, deploy). Sistem bira optimalni UAV, PVO, ZZ sistem, salje ga na poziciju presretanja uz realno-vremensko azuriranje trajektorije. Kad meta udje u domet, dvostepena potvrda operatera aktivira angazovanje. Feedback petlja evaluira rezultat: pogodak zatvara misiju, promasaj pokrece ponovni proracun trajektorije.

## 1.3 Scope

U scopu sistema:

- Softverska arhitektura zasnovana na 3 kontrolera

- UML modelovanje kompletnog sistema (10 dijagram tipova)

- Obrada i fuzija heterogenih senzorskih podataka

- AI/ML identifikacija i klasifikacija pretnji

- Procjena trajektorije i tačke presretanja (Kalman filter)

- Sistem alarma i upravljanje odbrambenim odgovorom

- Koordinacija autonomnih vozila (UAV/UGV) za presretanje

- Operator decision interface s feedback petljom

Van scopa:

- Firmware za UAV/UGV hardver

- Fizicki hardver i elektronika senzora

- ML model trening (pretpostavljamo pre-trenirane modele)

- Administratorski panel i upravljanje korisnicima

Quality Attributes:

| Atribut | Zahtjev |
| --- | --- |
| Latencija | < 200ms od detekcije do prikaza operateru |
| Raspoloživost | 99.9% uptime (manje od 8.7h godisnjeg downtime-a) |
| Skalabilnost | 50+ simultanih UAV/UGV jedinica |
| Pouzdanost | Automatski failover, redundantne komunikacione putanje |
| Sigurnost | AES-256-GCM enkripcija, mTLS, audit logging |

# II. UML DIJAGRAMI

## 2.1 Use Case Diagram

Use Case dijagram prikazuje funkcionalne zahtjeve sistema iz perspektive aktera. Identifikovana su tri glavna aktera i 12 use case-ova rasporedjenih po kontrolerima.

### Akteri

| Akter | Opis |
| --- | --- |
| Operator | Vojni operater koji prati zonu, pregleda alarme, odobrava ili odbacuje odbrambene akcije i procjenjuje rezultate |
| Admin | Sistemski administrator odgovoran za konfiguraciju sistema, upravljanje korisnickim nalozima i pracenje sistemskog zdravlja |
| Security Analyst | Analiticar sigurnosti koji pregledava audit logove, analizira obrasce prijetnji i optimizuje pravila detekcije |

### Use Case-ovi

| ID | Naziv | Akter | Kontroler |
| --- | --- | --- | --- |
| UC01 | Detect Activity (Acoustic, Seismic, Radar, LiDAR, Camera) | Sistem | Controller 1 |
| UC02 | Pre-Process Sensor Data | Sistem | Controller 1 |
| UC03 | Monitor Zone (UGV/UAV Heading) | Operator | Controller 1 |
| UC04 | Fuse Pre-Processed Sensor Data | Sistem | Controller 2 |
| UC05 | AI Target Identification | Sistem | Controller 2 |
| UC06 | Estimate Target Trajectory | Sistem | Controller 2 |
| UC07 | Generate Alarm (Threat Detected) | Sistem | Controller 2 |
| UC08 | Review Alarm & Target Data | Operator | Controller 3 |
| UC09 | Approve / Deny Action | Operator | Controller 3 |
| UC10 | Activate Defence System (Wakeup) | Sistem | Controller 3 |
| UC11 | Deploy UGV/UAV to Intercept Position | Sistem | Controller 3 |
| UC12 | Assess Action Feedback (Hit/Miss) | Operator | Controller 3 |

Tok sistema (sekvencijalni redosljed):

- Senzori detektuju aktivnost (akustika, seizmika, radar, LiDAR, kamera)

- Edge procesor pre-procesira podatke i filtrira sum

- UGV/UAV heading monitoring — pracenje pozicija autonomnih vozila

- Sensor Fusion + AI identifikacija mete (confidence > 0.85)

- Procjena trajektorije i tacke presretanja

- Alarm generisan — operater prima obavjestenje

- Operater odobrava ili odbacuje akciju (APPROVE/DENY)

- Odbrambeni sistem wakeup — ulazak u Get Ready mod

- UGV/UAV vozi/leti do pozicije presretanja

- Meta u dometu — operater potvrdjuje angazovanje

- Pogodak: misija zavrsena | Promasaj: ponovni proracun trajektorije

## 2.2 Class Diagram

Class dijagram modeluje staticki strukturni pogled na sistem. Sistem je organizovan u 7 paketa koji sadrze ukupno 20 klasa sa svim atributima, metodama i medjusobnim relacijama.

### Paketi i klase

| Paket | Klase |
| --- | --- |
| User Management | Operator |
| Command & Control | CommandCenter, Command, MissionController |
| Autonomous Vehicles | Device (abstract), UAV, UGV |
| Sensor Layer | Sensor (abstract), AcousticSensor, SeismicSensor, RadarSensor, LiDARSensor, CameraSensor, SensorFusionEngine |
| Threat Detection & Tracking | ThreatDetectionModule, TargetTracker, TrajectoryEstimator |
| Defence System | DefenceController, AlarmSystem, DefenceAction, ActionFeedback |
| Infrastructure | EncryptionService, AuditLog |

### Kljucne klase — atributi i metode

| Klasa | Atributi | Metode |
| --- | --- | --- |
| Operator | operatorId: String, username: String, assignedZone: String, clearanceLevel: Integer, lastLogin: DateTime | login(credentials): AuthToken, logout(): void, monitorZone(): ZoneStatus, reviewAlarm(alarm): AlarmDetails, approveAction(action): Boolean, denyAction(action): Boolean, assessFeedback(feedback): Decision |
| Device (abstract) | deviceId: String, type: DeviceType, role: DeviceRole, status: DeviceStatus, location: GeoCoordinate, batteryLevel: Float | connect(): Boolean, disconnect(): void, sendTelemetry(): TelemetryData, receiveCommand(cmd): void |
| UAV | altitude: Float, speed: Float, heading: Float | takeOff(altitude): void, land(): void, setWaypoints(waypoints): void, flyToInterceptPosition(pos): void |
| UGV | terrainMode: TerrainMode, speed: Float, obstacleDetected: Boolean | navigate(destination): void, stop(): void, driveToInterceptPosition(pos): void |
| SensorFusionEngine | sensors: List<Sensor>, fusionAlgorithm: FusionAlgorithm | fuseData(dataSet): FusedData, detectAnomalies(data): FusedData, applyKalmanFilter(data): SensorData |
| ThreatDetectionModule | rules: List<DetectionRule>, mlModel: ThreatModel | analyzeData(data): List<Threat>, classifyThreat(threat): ThreatLevel, identifyTarget(data): TargetIdentification |
| TrajectoryEstimator | kalmanFilter: KalmanFilter | estimateTrajectory(track): Trajectory, predictInterceptPoint(traj): GeoCoordinate, recalculateTrajectory(track, feedback): Trajectory |
| DefenceController | status: DefenceStatus, assignedDevices: List<Device> | wakeup(): void, getReady(target): void, deployToIntercept(device, position): void, executeAction(action): ActionResult, standDown(): void |

### Relacije

| Tip relacije | Opis |
| --- | --- |
| Nasljedivanje (extends) | UAV, UGV nasljeduju apstraktnu klasu Device |
| Nasljedivanje (extends) | AcousticSensor, SeismicSensor, RadarSensor, LiDARSensor, CameraSensor nasljeduju apstraktnu klasu Sensor |
| Agregacija (1..*) | Device agregira Sensor — jedan uredaj moze imati vise senzora |
| Asocijacija (->) | SensorFusionEngine → ThreatDetectionModule → TargetTracker → TrajectoryEstimator |
| Asocijacija (->) | AlarmSystem → DefenceController → DefenceAction → ActionFeedback |
| Asocijacija (->) | Operator odobrava/odbacuje DefenceAction (dvostepena autorizacija) |
| Zavisnost (..>) | ActionFeedback → TrajectoryEstimator (promasaj trigguje ponovni proracun) |
| Koristenje | EncryptionService koriste Command i CommandCenter za enkripciju komunikacije |
| Koristenje | AuditLog koriste svi servisi za belezenje kriticnih dogadjaja |

## 2.3 Sequence Diagrams

Sequence dijagrami modeluju dinamicke interakcije izmedju objekata u sistemu tokom kljucnih scenarija. Definisana su 4 glavna scenarija koji pokrivaju kompletan zivotni ciklus prijetnje.

### Scenario 1: Detekcija aktivnosti i pre-procesiranje (Controller 1)

Ovaj scenario opisuje paralelno prikupljanje podataka sa svih senzora, edge pre-procesiranje na UAV/UGV platformi i slanje na centralni server.

Ucesnici: AcousticSensor, SeismicSensor, RadarSensor, LiDARSensor, CameraSensor, EdgeProcessor, Controller2

- AcousticSensor i SeismicSensor detektuju signal (rano otkrivanje — zvuk/vibracije)

- RadarSensor mjeri udaljenost i velocitet mete

- LiDARSensor generise 3D point cloud okoline

- CameraSensor hvata video frame i vrsi detekciju objekata

- EdgeProcessor prima raw podatke od svih senzora paralelno

- EdgeProcessor filtrira sum, normalizuje podatke i vrsi prvu klasifikaciju

- EdgeProcessor salje pre-procesirane SensorData pakete Controller2

### Scenario 2: Fuzija senzora i identifikacija mete (Controller 2)

Ovaj scenario opisuje primjenu Kalman filtera, AI/ML identifikaciju i generisanje alarma.

Ucesnici: SensorFusionEngine, ThreatDetectionModule, TargetTracker, TrajectoryEstimator, AlarmSystem, Operator

- SensorFusionEngine prima pre-procesirane pakete od Controller1

- Primjena Kalman filtera za fuziju heterogenih mjerenja

- Detekcija anomalija i agregacija detekcija u jedinstveni FusedData objekat

- ThreatDetectionModule analizira FusedData koristeci ML model

- Identifikacija mete — ako confidence > 0.85, klasifikacija prijetnje

- TargetTracker inicijalizuje track objekat za pracenje mete

- TrajectoryEstimator procjenjuje trajektoriju i izracunava tacku presretanja

- AlarmSystem generise Alarm objekat (HIGH priority) i salje obavjestenje operateru

### Scenario 3: Odluka operatera i odbrambena akcija (Controller 3)

Ovaj scenario opisuje pregled alarma, donošenje odluke i koordinaciju odbrambene akcije.

Ucesnici: Operator, AlarmSystem, DefenceController, MissionController, UAV/UGV, TrajectoryEstimator

- Operator prima alarm i ucitava detalje o meti (lokacija, brzina, heading, threat level)

- Operator pregleda procijenjenu trajektoriju i tacku presretanja na Map Viewer-u

- Operator donosi odluku: APPROVE ili DENY

- Kod APPROVE: DefenceController prima nalog za aktivaciju

- DefenceController: wakeup() → getReady(target) → selectOptimalDevice()

- MissionController dodjeljuje UAV/UGV i generise misiju presretanja

- UAV/UGV prima waypoints i krece prema poziciji presretanja

- TrajectoryEstimator kontinuirano azurira procjenu dok se uredaj priblizava

- Kad meta udje u domet — drugi confirmation od operatera za angazovanje

- DefenceController salje executeAction() komandu — angazovanje

### Scenario 4: Feedback petlja — Pogodak / Promasaj

Ovaj scenario opisuje evaluaciju rezultata akcije i feedback petlju za iterativni odbrambeni odgovor.

Ucesnici: ActionFeedback, TargetTracker, TrajectoryEstimator, AlarmSystem, DefenceController, AuditLog

- ActionFeedback prima rezultat angazovanja od senzora na terenu

- POGODAK (HIT): TargetTracker.lostTrack() — pracenje zatvoreno

- POGODAK: AlarmSystem.resolveAlarm() — alarm razrjesen

- POGODAK: DefenceController.standDown() — uredaj povucen, misija zavrsena

- POGODAK: AuditLog belezi MISSION_ACCOMPLISHED s vremenskim pecat

- PROMASAJ (MISS): ActionFeedback trigguije TrajectoryEstimator.recalculate()

- PROMASAJ: TargetTracker.updateTrack() — azuriranje podataka o meti

- PROMASAJ: TrajectoryEstimator primjenjuje prilagodjeni Kalman s novim podacima

- PROMASAJ: Nova DefenceAction predlozena operateru — ciklus se ponavlja

- GUBITAK METE: TargetTracker.lostTrack() — status TARGET_LOST, alarm suspendovan

## 2.4 Activity Diagrams

Activity dijagrami modeluju tok kontrole i podataka kroz procese sistema. Koriste swim lanes za jasno prikazivanje odgovornosti svake komponente.

### Proces 1: Kompletni tok misije (Mission Workflow)

Swim lanes: Controller 1: Sensor Detection | Controller 2: Sensor Fusion | Controller 3: Operator Decision | Audit Log

| Swim Lane | Aktivnosti |
| --- | --- |
| Controller 1 (Sensor Detection) | Start → Detect Activity → Pre-Process Data → Monitor Zone → [Fork] → Send to Controller 2 |
| Controller 2 (Sensor Fusion) | Receive Data → Apply Kalman Filter → AI Identify Target → [Decision: confidence > 0.85?] → YES: Estimate Trajectory → Generate Alarm ¦ NO: Continue monitoring |
| Controller 3 (Operator Decision) | Receive Alarm → Review Target Data → [Decision: APPROVE/DENY] → APPROVE: Wakeup Defence → Deploy → In Range? → Engage ¦ DENY: Log & Resume |
| Audit Log | Log Detection Event → Log Alarm Generated → Log Operator Decision → Log Action Result → [Decision: HIT/MISS] → HIT: Log Mission Complete ¦ MISS: Log Recalculate |

### Proces 2: Defence Action Feedback (Hit / Miss)

Swim lanes: Defence Controller | Target Tracking | Operator | Alarm System | Audit Log

| Swim Lane | Aktivnosti |
| --- | --- |
| Defence Controller | Execute Action → Receive Feedback → [HIT/MISS/LOST?] → HIT: StandDown ¦ MISS: Recalculate Order ¦ LOST: Suspend |
| Target Tracking | Update Track → [HIT: Close Track] ¦ [MISS: Recompute Trajectory + Kalman Update] ¦ [LOST: Mark TARGET_LOST → Deactivate Tracking] |
| Operator | Receive Result Notification → [HIT: Review Summary → Confirm Complete] ¦ [MISS: Review New Proposal → Approve/Deny] ¦ [LOST: Acknowledge] |
| Alarm System | [HIT: Resolve Alarm → Archive] ¦ [MISS: Update Alarm Status → Re-alert Operator] ¦ [LOST: Suspend Alarm] |
| Audit Log | Log Feedback Result → Log Track Status Change → Log Alarm Resolution → Log Operator Acknowledgement |

## 2.5 State Machine Diagrams

State Machine dijagrami modeluju zivotni ciklus kljucnih objekata u sistemu prikazujuci sva moguca stanja i tranzicije izmedju njih.

### State Machine 1: Odbrambeni sistem (Defence System States)

Ovaj dijagram modeluje stanja kompletnog odbrambenog sistema od prvog signala do zavrsetka misije.

Stanja i tranzicije:

| Stanje | Opis | Tranzicije |
| --- | --- | --- |
| Monitoring | Kontinuirani nadzor zone. Senzori aktivni, sistem u stand-by. | ActivityDetected signal → prelaz u ActivityDetected |
| ActivityDetected | Senzori detektovali signal, pre-processing i fuzija u toku. | AI confidence > 0.85 → TargetConfirmed ¦ confidence <= 0.85 → vracanje na Monitoring |
| TargetConfirmed | AI potvrdio metu, generisanje internih podataka o prijetnji. | Automatski → AlarmGenerated |
| AlarmGenerated | Alarm kreiran i proslijedjen operateru. Ceka se odluka. | Operator.APPROVE → DefenceReady ¦ Operator.DENY → Monitoring |
| DefenceReady | Kompozitno stanje: Wakeup → GetReady → Deploying. Sistem se aktivira. | Uredaj na poziciji → InRange ¦ Timeout/Abort → StandDown |
| InRange | UAV/UGV u dometu mete. Ceka se finalna potvrda operatera. | Operator.CONFIRM → Engaged ¦ Operator.ABORT → StandDown |
| Engaged | Akcija angazovanja u toku. | HIT → MissionAccomplished ¦ MISS → Recalculating ¦ TARGET_LOST → StandDown |
| MissionAccomplished | Pogodak potvrddjen. Uredaj se povlaci, alarm razrjesen. | → Monitoring (reset) |
| Recalculating | Promasaj. Nova trajektorija se racuna, nova akcija predlozena. | New trajectory ready → AlarmGenerated (novi ciklus) |

### State Machine 2: Device Connection States

Ovaj dijagram modeluje zivotni ciklus konekcije UAV/UGV uredjaja prema centralnom sistemu.

| Stanje | Opis | Tranzicije |
| --- | --- | --- |
| Offline | Uredaj iskljucen ili nema napajanja. | PowerOn / ConnectRequest → Connecting |
| Connecting | Pokusaj uspostavljanja konekcije (max 5 pokusaja). | Success → Online ¦ Failure (max retries) → Error |
| Online | Konekcija uspostavljena. Uredaj registrovan u sistemu. | Receive Command → Transmitting ¦ No activity (timeout) → Idle |
| Idle | Konekcija aktivna ali nema transfera podataka. | Activity → Transmitting ¦ LostSignal → Connecting (retry) |
| Transmitting | Aktivni transfer — slanje telemetrije ili primanje komandi. | Transfer complete → Online ¦ LostSignal → Connecting |
| Error | Kriticna greska u komunikaciji. | Reset → Offline ¦ Auto-recover → Connecting |

## 2.6 Component Diagram

Component dijagram prikazuje arhitekturu sistema kao skup komponenti sa eksplicitnim interfejsima i zavisnostima. Sistem je organizovan u 5 horizontalnih slojeva.

| Sloj | Komponente | Interfejsi |
| --- | --- | --- |
| Presentation Layer | Operator Dashboard UI, Real-Time Map Viewer, Alarm Console | REST/WebSocket (HTTPS/TLS 1.3) |
| Application Layer | API Gateway (nginx), Command & Control Service, Mission Controller Service, Alarm Service, Defence Controller Service | REST API, WebSocket, gRPC |
| Processing Layer | Sensor Fusion Engine, Threat Detection Module, Target Tracking Service, Trajectory Estimator, AI/ML Inference Engine (ONNX) | gRPC (mTLS), Internal Message Bus |
| Device Layer | UAV Communication Adapter, UGV Communication Adapter, Device Registry, Edge Processor | Encrypted Radio (AES-256-GCM), Device Protocol |
| Infrastructure Layer | Encryption Service, Audit Log Service, PostgreSQL (Mission DB), TimescaleDB (Sensor Data), MongoDB (Audit DB) | JDBC/TCP, Internal API |

### Kljucni interfejsi

| Interfejs | Protokol | Namjena |
| --- | --- | --- |
| REST / WebSocket | HTTPS (TLS 1.3) — Port 443 | Presentation ↔ Application Layer komunikacija |
| gRPC | mTLS — Port 50051 | Application ↔ Processing Layer komunikacija |
| Device Protocol | AES-256-GCM Encrypted Radio | Application ↔ Field Device komunikacija |
| Database | JDBC/TCP — interni network | Application/Processing ↔ Infrastructure |
| Message Bus | Async internal pub/sub | Interna komunikacija izmedju Processing servisa |

## 2.7 Deployment Diagram

Deployment dijagram prikazuje fizicku arhitekturu sistema — raspodjelu softverskih artefakata po hardverskim cvorovima i komunikacione veze izmedju njih.

### Command Center HQ (Sigurni objekat)

| Cvor | OS/Platforma | Artefakti |
| --- | --- | --- |
| Operator Workstation | Linux PC | Operator Dashboard (React Browser App), Alarm Console, Map Viewer |
| Application Server | Linux — Docker Compose | API Gateway (nginx), Command & Control Service (FastAPI), Mission Controller Service (FastAPI), Alarm Service (FastAPI), Defence Controller Service (FastAPI), Encryption Service (Python) |
| Data Server | Linux | PostgreSQL (Mission DB), TimescaleDB (Sensor Time-Series), MongoDB (Audit Logs) |
| AI Processing Server | Linux — GPU (CUDA) | Sensor Fusion Engine (Python/NumPy), Threat Detector (ONNX Runtime), Target Tracker (Python/SciPy), Trajectory Estimator (Python/SciPy) |

### Field Operations Zone

| Cvor | OS/Platforma | Artefakti |
| --- | --- | --- |
| UAV Unit | Embedded Linux — ARM | UAV Firmware (ROS2 Node), Camera Driver (OpenCV), Radar Driver (Custom C++), LiDAR Driver (ROS2 PointCloud2), Acoustic Sensor Driver, Edge Processor (ONNX Runtime int8) |
| UGV Unit | Embedded Linux — x86 | UGV Firmware (ROS2 Node), Sensor Array Driver (ROS2), Seismic Sensor Driver, Edge Processor (ONNX Runtime int8) |

### Komunikacioni linkovi

| Izvor | Odrediste | Protokol |
| --- | --- | --- |
| Operator Workstation | Application Server | HTTPS / WSS (TLS 1.3 — Port 443) |
| Application Server | Data Server | JDBC / TCP (interni network) |
| Application Server | AI Processing Server | gRPC (mTLS) |
| AI Processing Server | Data Server | JDBC / TCP (interni network) |
| UAV/UGV | Application Server | Encrypted Radio Link (AES-256-GCM) |

## 2.8 Package Diagram

Package dijagram prikazuje organizaciju sistema u module/pakete i zavisnosti izmedju njih. Sistem je organizovan po principu Layered Architecture s jasnom hijerarhijom zavisnosti.

| Paket | Puni naziv | Sadrzaj | Zavisnosti |
| --- | --- | --- | --- |
| presentation | amcs.presentation | Operator Dashboard, Alarm Console, Map Viewer, Trajectory Overlay UI | → amcs.api |
| api | amcs.api | API Gateway, REST Controllers, WebSocket Handlers, DTO klase, Request/Response modeli | → amcs.domain |
| domain | amcs.domain | Command, Device (UAV/UGV), Sensor, User (Operator), Mission, Alarm — domenski entiteti | → amcs.processing, amcs.infrastructure |
| processing | amcs.processing | SensorFusionEngine, ThreatDetectionModule, TargetTracker, TrajectoryEstimator, AI/ML Engine | → amcs.infrastructure |
| defence | amcs.defence | DefenceController, AlarmSystem, DefenceAction, ActionFeedback — odbrambena logika | → amcs.processing, amcs.domain, amcs.infrastructure |
| infrastructure | amcs.infrastructure | Persistence (PostgreSQL, TimescaleDB, MongoDB), EncryptionService, AuditLog, Repository interfejsi | – (leaf paket) |

Hijerarhija zavisnosti (odgore prema dolje):

amcs.presentation → amcs.api → amcs.domain → amcs.processing → amcs.infrastructure
amcs.defence → amcs.processing / amcs.domain / amcs.infrastructure

Princip koji se potuje: visi slojevi zavise od nizih, ali nizi slojevi nikada ne zavise od visih (Dependency Inversion Principle). Krozni zavisnosti su eksplicitno zabranjene.

## 2.9 Object Diagram

Object dijagram prikazuje konkretan runtime snapshot sistema tokom aktivne misije MSN-2026-003. Prikazane su instance klasa s konkretnim vrijednostima atributa u trenutku odbrambene akcije.

### Kontekst snimka: Aktivna misija MSN-2026-003

Vremenski peckt: 2026-03-21 14:34:12Z. Status: Odbrambena akcija u toku — UAV Alpha i UGV Bravo deploying prema tacki presretanja. Alarm odobren od Operatera Jovic.

| Instanca | Klasa | Kljucni atributi |
| --- | --- | --- |
| uav_alpha | UAV | deviceId='UAV-001', status=TRANSMITTING, altitude=120m, speed=85km/h, battery=78.5% |
| ugv_bravo | UGV | deviceId='UGV-001', status=DEPLOYING, speed=25km/h, terrainMode=OFF_ROAD, battery=92.3% |
| cam_01 | CameraSensor | sensorId='CAM-001', resolution=1080p, fps=30, nightVision=true, mountedOn=uav_alpha |
| radar_01 | RadarSensor | sensorId='RAD-001', range=5000m, frequency=77GHz, mountedOn=uav_alpha |
| lidar_01 | LiDARSensor | sensorId='LDR-001', pointsPerSecond=100K, range=200m, mountedOn=ugv_bravo |
| acoustic_01 | AcousticSensor | sensorId='ACU-001', frequencyRange=20Hz-20kHz, isStandalone=true |
| alarm_001 | Alarm | alarmId='ALM-001', level=HIGH, status=OPERATOR_APPROVED, confidence=93%, targetId='TGT-001' |
| action_001 | DefenceAction | actionId='ACT-001', type=INTERCEPT, status=DEPLOYING, assignedDevices=[uav_alpha, ugv_bravo] |
| track_001 | Track | trackId='TRK-001', velocity=15m/s, heading=180°, lastUpdate=14:34:10Z |
| trajectory_001 | Trajectory | interceptPoint=GeoCoord(44.123, 20.456), ETA=14:35:00Z, confidence=0.91 |
| op_jovic | Operator | username='jovic', clearanceLevel=3, assignedZone='ZONE-BRAVO', lastLogin=14:00:00Z |
| mission_03 | MissionController | missionId='MSN-2026-003', status=ACTIVE, startTime=14:30:00Z, assignedDevices=[uav_alpha, ugv_bravo] |

# III. ZAKLJUCAK

## 3.1 Arhitekturne odluke

AMCS je projektovan kao socijabilna, bezbjedna i skalabilna softverska arhitektura namijenjena za kriticne borbene sisteme. Kljucne arhitekturne odluke su:

Troslojna separacija briga (3 kontrolera)

Jasna podjela izmedju detekcije, obrade i odlucivanja eliminise ogranicenja monolitnih sistema i olaksava nezavisni razvoj i testiranje.

Edge Computing na UAV/UGV

ONNX Runtime na ARM/x86 embedded Linux smanjuje bandwidth prema centralnom serveru za ~80% i osigurava rad i pri privremenom gubitku konekcije.

AI/ML identifikacija meta

ONNX Runtime za inference na edge i GPU server za kompleksnu analizu. Prag pouzdanosti od 0.85 balansira sensitivnost i specificnost.

Feedback petlja (promasaj → ponovni proracun)

Adaptivni Kalman filter koji prima feedback iz akcionih rezultata omogucava iterativni odbrambeni odgovor bez ponovnog pokretanja cijelog procesa.

Covjek-u-petlji (Human-in-the-Loop)

Dvostepena potvrda operatera (approve inicijalna akcija + confirm engagement) osigurava eticki i pravni framework autonomnih borbenih sistema.

End-to-end enkripcija

TLS 1.3 za web komunikaciju, mTLS za servis-servis komunikaciju, AES-256-GCM za radio linkove eliminisu rizik od presretanja i manipulacije komandama.

## 3.2 Postignuti ciljevi i quality attributes

| Quality Attribute | Zahtjev | Postignuto rijesenje |
| --- | --- | --- |
| Latencija | < 200ms | Edge processing, in-memory Kalman, gRPC umjesto REST za interni processing |
| Raspolozivost | 99.9% uptime | Docker, automatski failover, redundantne komunikacione putanje |
| Skalabilnost | 50+ uredjaja | Adapter pattern za device layer, stateless processing servisi, TimescaleDB |
| Pouzdanost | Auomatski failover | State machine za konekciju (retry logic), track loss handling |
| Sigurnost | End-to-end | AES-256-GCM, TLS 1.3, mTLS, audit logging, role-based access (clearanceLevel) |

## 3.3 Buduce aktivnosti

- Backend implementacija — Python/FastAPI, gRPC servisi, Docker Compose orkestracija

- CI/CD pipeline — GitHub Actions, automated testing, container registry

- ML modeli za fuziju i identifikaciju — PyTorch trening, ONNX export, kvantizacija za edge

- ROS2 + PoC simulacija — Gazebo simulator, virtuelni UAV/UGV sa senzorskim driverima

- Zavrsni rad: integracija kompletnog sistema, evaluacija latencije i pouzdanosti na simuliranom scenariju

## 3.4 Literatura

[1] Gamma, E., Helm, R., Johnson, R., Vlissides, J. Design Patterns: Elements of Reusable Object-Oriented Software. Addison-Wesley, 1994.

[2] Fowler, M. UML Distilled: A Brief Guide to the Standard Object Modeling Language. 3rd ed., Addison-Wesley, 2003.

[3] Bass, L., Clements, P., Kazman, R. Software Architecture in Practice. 3rd ed., Addison-Wesley, 2012.

[4] Mahler, R., et al. Multi-Sensor Fusion for Autonomous Vehicle Perception. IEEE Transactions on Intelligent Transportation Systems, 2021.

[5] OMG. UML Specification v2.5.1. Object Management Group, 2017.

[6] Welch, G., Bishop, G. An Introduction to the Kalman Filter. University of North Carolina, 2006.

[7] Thrun, S., Burgard, W., Fox, D. Probabilistic Robotics. MIT Press, 2005.